Summary: This report provides a line-by-line, physics-aware explanation of the Task 1 Python code for computing drone (rotorcraft) energy and maximum range. We explain the imports, constants, and each function (total_energy and calculate_distance) in detail, mapping them to the underlying flight physics. We derive and justify the energy formula components; particularly the unusual m1.5/ρ term; using rotorcraft induced power theory[1][2]. We discuss the assumptions and limitations (constant ascent speed, neglect of cruise energy in the vertical segment, etc.) and note how the empirical constants b0,b1 would be calibrated via flight tests or wind-tunnel data. We then show how Task 1 integrates with Tasks 2–4 (how its outputs feed into path planning and range-checking logic). The writeup includes a concise spoken-style script (≈5 minutes of explanation) and a longer written exposition suitable for a technical blog. We also suggest figures (e.g. a block diagram of flight phases, a derivation flowchart, and an energy-vs-mass sensitivity plot) and provide FAQs that pre-empt likely exam/viva questions. Finally, we provide citations to rotorcraft/aircraft aerodynamics references to justify the model.

## Code Walkthrough

Below is the Task 1 code, annotated line by line with detailed explanations:

# Task 1
# 100500474
import math

# define b1 and b0
b1_cruise = 68.9  
b0_cruise = 16.8

b1_takeoff = 80.4
b0_takeoff = 13.8

b1_landing = 71.5
b0_landing = -24.3

- import math: Imports Python’s math library to use mathematical functions (here, math.sqrt and any others needed).

- Constants (b0, b1): These six numbers are empirical model parameters (with units of Watts). The b1 terms scale with m1.5/ρ (see below) and the b0 terms are constant offsets (often from profile losses). They differ for each flight phase:

- b1_takeoff = 80.4, b0_takeoff = 13.8: coefficients for take-off (vertical climb).

- b1_landing = 71.5, b0_landing = -24.3: coefficients for landing (vertical descent).

- b1_cruise = 68.9, b0_cruise = 16.8: coefficients for cruise (horizontal flight).
These values would typically come from fitting a power model to flight test data or high-fidelity simulations. (Units note: when plugged into the formula below, b1 has units so that b1m1.5/ρ yields power in Watts, and b0 is in Watts.)

# total energy (a)
def total_energy(m, p, h, V, mode):                                                

    if mode == "takeoff":
        energy_val = (b1_takeoff * (m**1.5 / math.sqrt(p)) + b0_takeoff) * (h / V)
    elif mode == "landing":
        energy_val = (b1_landing * (m**1.5 / math.sqrt(p)) + b0_landing) * (h / V)
    else:
        energy_val = 0

    return energy_val

- Function total_energy(m, p, h, V, mode): Computes the energy (in joules) used in a vertical segment of flight (takeoff or landing). Parameters:

  - m (mass, in kg)

  - p (air density, in kg/m³)

  - h (vertical distance climbed or descended, in meters)

  - V (vertical speed during that segment, in m/s)

  - mode (string, either "takeoff" or "landing")

The code does: 1. Checks mode. If "takeoff", it uses the takeoff coefficients; if "landing", the landing coefficients; otherwise it returns 0 (invalid mode yields zero energy).
2. Inside each branch: It calculates

power=b1m1.5p+b0,

then multiplies by (h/V). Since h/V is time (meters divided by m/s = seconds), this yields energy (Watts × seconds = Joules). - The term m1.5/p comes from rotorcraft induced-power theory (see next section). Multiplying by b1 (with units of W·√ρ/kg^1.5) and adding b0 (W) gives power in watts during vertical motion. - Thus energy_val = power * time. 3. Returns the energy (in joules).

# define the maximum range (b)
def calculate_distance(Vcr, m, p, Emax, h_takeoff, h_landing, V_takeoff, V_landing):

    # energy used in the vertical phases
    energy_takeoff = total_energy(m, p, h_takeoff, V_takeoff, "takeoff")
    energy_landing = total_energy(m, p, h_landing, V_landing, "landing")

    numerator = (Emax - (energy_takeoff + energy_landing)) * Vcr
    denominator = (b1_cruise * (m**1.5 / math.sqrt(p)) + b0_cruise)

    d_cruise = numerator / denominator
    return d_cruise

- Function calculate_distance(...): Computes maximum horizontal cruise distance the drone can fly, given total available energy Emax. Parameters:

  - Vcr - cruise speed (m/s)

  - m - mass (kg)

  - p - air density (kg/m³)

  - Emax - total energy capacity (Joules, e.g. battery energy)

  - h_takeoff, h_landing - vertical distances of takeoff and landing (m)

  - V_takeoff, V_landing - vertical speeds during takeoff/landing (m/s)

Steps: 1. Compute vertical energies: Calls total_energy to find energy used in takeoff and landing: - energy_takeoff = total_energy(...) for ascent of height h_takeoff. - energy_landing = total_energy(...) for descent of height h_landing. 2. Available cruise energy: Subtract these vertical energies from total energy:

Eavail=Emax-Etakeoff+Elanding.

3. Cruise distance: We assume the remaining energy goes entirely into horizontal cruise at speed Vcr. If cruise power is assumed constant and equal to Pcruise=b1,cruise m1.5/p+b0,cruise, then the time available is t=Eavail/Pcruise. The distance is d=Vcr t. Algebraically:

dcruise=EavailPcruise Vcr=Emax-Etakeoff-Elanding Vcrb1,cruise m1.5/p+b0,cruise.

The code implements this as numerator = (Emax - (energy_takeoff + energy_landing)) * Vcr and denominator = ..., and returns numerator/denominator. Units check: (J * m/s) / (W) = m.

In summary, calculate_distance returns the horizontal range (meters) achievable with the given total energy, after accounting for vertical energy costs.

Lastly, the code prints results for predefined scenarios and accepts user input for a custom scenario. (We omit that boilerplate in this explanation, focusing on the model itself.)

## Induced Power Theory & Energy Formula

The core model in Task 1 is semi-empirical: Power ≈ b1 m1.5/ρ+b0. Where does that come from?

### Induced Power in Hover

In rotorcraft theory, induced power is the power needed to generate lift (thrust) at the rotor, primarily due to creating a downward momentum (see momentum theory). For a hover or vertical climb, the ideal induced power is P_{\rm induced,ideal} = T\,v_i, where T is thrust (equal to weight mg in steady hover) and vi is the induced velocity through the rotor disk. Momentum theory gives vi=T2ρA, where ρ is air density and A is rotor disk area. Substituting T=mg yields: P_{\rm induced,ideal} = T \sqrt{\frac{T}{2\rho A}} = (m g)\sqrt{\frac{m g}{2\rho A}}.

This means $P_{\rm induced,ideal} \propto \frac{m^{1.5}}{\sqrt{\rho}}$ (since g, A, and 2 are constants). In other words, the induced power scales with m3/2 and inversely with ρ. In the NDARC rotorcraft handbook, this is stated as P=T vi with vi=T/2ρA[1]. Our code’s use of m1.5/p (with p=ρ) directly reflects this physics: heavier drones (larger m) or lower density (ρ) demand much more power (superlinear in m).

Citation: “The ideal induced power is P_{\rm ideal} = T\,v_i$”[1], with vi=T/2ρA. For thrust T=mg, this gives P_{\rm ideal}\propto m^{3/2}/\sqrt{\rho}.

The paper “Energy Consumption Models for a Single-Rotor UAV”[2] (among others) explicitly shows a hover power model with a term ∝W3/2/ρ, where W=mg. Equation (1) there is

P=P0+1+kW3/22ρA ,

which is exactly a constant P0 plus a factor times W1.5/ρ[2]. In our code, b1 plays the role of 1+k/2A (and includes the factors like g1.5 implicitly), and b0 corresponds to a base power P0.

### Semi-Empirical Terms b0,b1

The model in Task 1 is semi-empirical: it uses the theoretically-motivated dependence m1.5/ρ multiplied by a fitted coefficient b1, plus an offset b0. Why two terms?

- b1m1.5/ρ: Captures the induced power portion, which dominates hover/climb performance and scales as discussed.

- b0: Accounts for profile and parasitic power that is (approximately) constant or less dependent on weight. For example, blade profile drag and non-idealities add a roughly constant overhead power. In landing, interestingly, b0 is negative (-24.3 W). A negative b0 for descent may indicate that induced power decreases when descending (a portion of energy is recovered) or simply an empirical adjustment (some descent energy is less than hover). In practice, b0 is found by fitting data.

Each flight phase (takeoff, cruise, landing) has its own b1,b0 because the aerodynamics differ: takeoff and landing have vertical motion (and possibly different rotor pitch settings), cruise has forward flight with added profile and parasite drag. Note the cruise b1 is a bit smaller than takeoff’s, reflecting different lift distribution; cruise b0 is positive (parasitic drag), etc.

Units: If m is in kg and ρ in kg/m³, then m1.5/ρ has units kg1.5/kg0.5/m1.5=kg⋅m1.5. To make power (Watts = J/s), b1 must have units W/(kg·m^{1.5}). Then b1m1.5/ρ is in watts, to which we add b0 (watts). Multiplying by time (h/V, seconds) gives energy (joules).

A quick units table:

| Symbol | Units | Typical Value (order) | Role |
| --- | --- | --- | --- |
| m | kg | e.g. 5–50 | drone mass |
| ρ (p) | kg/m³ | ~1.225 at sea level | air density |
| V | m/s | e.g. 5 (vertical speed) | vertical/climb velocity |
| h | m | e.g. 10 -100 | vertical distance (climb/descent) |
| b1 | W/(kg·m^1.5) | e.g. 80 (see code) | induced power scale factor |
| b0 | W | e.g. 14 W | base/profile power |
| Energy | J | - | power (W) × time (s) |
| Distance d | m | - | horizontal range output |

### Deriving the Energy Formula

Putting it together for a vertical climb: if the instantaneous power is

Pm,ρ≈b1m1.5ρ+b0,

and the climb speed is constant V, climbing h meters takes time t=h/V. Then the energy used is E = P \cdot t = \Bigl(b_1\frac{m^{1.5}}{\sqrt{\rho}} + b_0\Bigr)\frac{h}{V}.

This is exactly how total_energy is coded for takeoff/landing. (For descent, a similar logic applies; a negative b0 can mean net energy recovery or lower power needed.)

The distance formula in calculate_distance follows by energy budget: remaining energy after vertical segments divided by cruise power. Cruise power is likewise modeled as Pcruise=b1,cruise m1.5/ρ+b0,cruise. The available cruise time is ΔE/Pcruise, so distance = Vcr×ΔE/Pcruise. This ignores any variable drag changes with speed - it assumes constant cruise power, which is a simplification (we could cite more complex cruise power laws from helicopter theory, but the code uses a constant power model via linearising to two terms).

### Assumptions and Limitations

- Constant climb/descent speed: The model uses h/V as time, implying the climb rate V is constant. Real drones may vary power to maintain speed, but assuming a fixed vertical speed simplifies the energy calc.

- No horizontal (cruise) energy in total_energy: The total_energy function only treats vertical phases. It does not compute cruise energy; cruise energy enters only in the distance formula as an average power. This assumes cruise power is roughly constant and captured by the cruise b1,b0.

- One-dimensional model: Wind, maneuvers, and efficiency losses are ignored. The model lumps all effects into b0 (profile/parasite drag) and b1.

- Empirical constants: The values of b1 and b0 must come from experiments. Typically, one would calibrate them by measuring power vs weight/density in controlled flights (e.g. hover tests at different masses and altitudes). For example, one could plot measured power vs m1.5/ρ and fit a line, giving b1 (slope) and b0 (intercept)[2]. The negative b0 for landing suggests a fitted decrease in power for descent (some drones can recover energy via motor regen when descending).

- Density variation: The model includes ρ (air density). If the drone changes altitude (density), the available range changes because induced power ∝1/ρ. The code uses the input p for density so users should supply correct values (e.g. sea level ~1.225 kg/m³).

- Zero and invalid values: If mode is not “takeoff” or “landing”, total_energy returns 0. The code does not explicitly check for impossible inputs (negative mass, zero speed, etc.), but any non-numeric input for speed or height would be caught by the try/except in Task 2 or by Python errors here.

In summary, this model is a simplified, linearized energy model grounded in rotorcraft induced power theory, with added empirical tuning. It’s not a full Navier–Stokes calculation, but it captures key trends and is cheap to compute in code.

## Integration with Tasks 2-4

Task 1 provides the energy and range calculations that feed into later tasks:

- Task 2 (Destinations Dictionary): Task 2 does not directly call Task 1, but it builds a dictionary of named locations. These destinations would be used in path planning. For instance, Task 4’s Drone class will load these destinations and then compute paths and check range; it will likely use Task 1’s energy model (via calculate_energy and calculate_range) to see if legs between destinations are feasible. Thus, the output of Task 1 (range given energy, or energy required) informs the drone’s decision on whether it can reach the next waypoint.

- Task 3 (Path Building): Task 3’s greedy path building algorithm computes distances between nodes using distance(a,b). Though it does not call Task 1, it uses a distance metric. Task 1 could interact by providing whether a proposed hop is within range or energy budget. In fact, one could imagine augmenting path_builder to check if the distance to the next node d is less than the maximum range from Task 1, rather than assuming a fixed drone_range. The drone_range parameter in Task 3 could be obtained by calling calculate_range from Task 1 logic for the current state.

- Task 4 (Drone Class): This final task unifies all logic. The Drone class’s methods calculate_energy and calculate_range are direct analogues of Task 1’s functions (but slightly modified to fit into a class). We see in Task 4 that check_trip calls calculate_energy(...) using the same formulas as in Task 1 (passing the b1,b0 constants). Also update_path in Task 4 effectively reconstructs a nearest-neighbor path (a variant of Task 3) and then check_trip uses the calculate_energy logic to decide if a leg exceeds the drone’s remaining energy, thereby marking recharge points. Thus, Task 1’s logic (energy from weight, density, distance) is the core of Task 4’s feasibility and recharge calculation.

In essence, Task 1 models the drone’s flight capability (how far it can go with a given battery), which informs path planning and range-checking in the later tasks. The interface is: Inputs: mass, density, speeds, heights, total energy. Outputs: energy needed for each phase and resulting maximum distance. Other tasks must supply the correct parameters (e.g. current mass, battery capacity). Error handling is minimal; the model will simply yield zero or negative distances if energy is insufficient.

Mermaid flowchart of data flow between tasks:

flowchart LR
    A[Task 2: Load Destinations] --> B[Destinations dict]
    B --> C[Task 4: Drone.destinations]
    C --> D[Task 4:update_path(origin,destination,range)]
    E[Task 1: calculate_range] --> D
    F[Task 3: build path] --> D
    D --> G[Task 4: current_path list]
    G --> H[Task 4: check_trip uses calculate_energy]

(This flowchart illustrates how destination data and the Task 1 energy functions feed into the final Drone planning.)

## Spoken Explanation (≈5 minutes)

Below is a concise, step-by-step “script” explanation of Task 1, as if you were presenting it orally:

“We start by importing Python’s math library for needed functions. Next, we define six constants – two for each flight phase: takeoff, cruise, and landing. These constants (b1 and b0) come from an empirical energy model. The core idea is that induced power in a rotor scales as m1.5/ρ, where m is the drone’s mass and ρ is air density[1]. This power is multiplied by a height-over-velocity term to get energy.

The function total_energy implements this model for vertical flight. It takes mass m, density p, vertical distance h, speed V, and the mode (‘takeoff’ or ‘landing’). It computes a power P=b1m1.5/p+b0 (in watts), then multiplies by time h/V (the duration of the climb or descent) to get energy in joules. If the mode is unrecognized, it returns 0. So total_energy returns the energy used to climb or descend a given height at given speed.

For horizontal cruise, the function calculate_distance uses an energy budget. It first computes energy for takeoff and landing via total_energy. Subtracting these from the battery’s total energy Emax gives the energy left for cruise. Assuming cruise power is constant P_{\rm cruise} = b_{1,\text{cruise}}\,m^{1.5}/\sqrt{p} + b_{0,\text{cruise}}, the available time is (E_{\max} - E_{\text{verticals}})/P_{\rm cruise}. Multiplying this time by cruise speed Vcr yields maximum cruise distance. In code, this is implemented as:

numerator = (Emax - (energy_takeoff + energy_landing)) * Vcr
denominator = (b1_cruise * (m**1.5 / math.sqrt(p)) + b0_cruise)
d_cruise = numerator / denominator

Here, (h/V) in the vertical case and numerator/denominator in the cruise case ensure unit consistency: power (W) times time (s) gives energy (J), and energy divided by power times speed gives distance (m).

The key physics behind the m1.5/ρ term comes from momentum theory. For a rotor to hover or climb, it must impart momentum to the air. The ideal induced power is P=TT/2ρA, where T=mg is thrust[1]. Substituting T yields a proportionality m1.5/ρ. The coefficients b1 and b0 are then tuned so that the formula matches real flight data[2]. For example, in hover we might measure power vs weight and fit a line to determine b1 and b0. Once calibrated, these constants encode all the complex losses (blade drag, motor inefficiency, etc.).

Finally, we run some test cases and allow user input to calculate a custom distance. In practice, this Task 1 model feeds into the path planning of later tasks: the computed range tells us which hops between destinations are feasible given the drone’s battery.

In summary, Task 1’s code cleanly separates vertical energy (with total_energy) from horizontal range (with calculate_distance). Each line implements a physics formula: induced-power scaling, time = distance/speed, and simple arithmetic to get energy or distance. The result is an easy-to-compute range estimator based on a helicopter-like energy model. (If asked, I’d note limitations: this assumes constant vertical speed, no wind, and uses fitted constants. Those would have to be explained if probed.)”

## Tables and Figures

Table 1: Key Code Lines and Explanations

| Code Snippet | Explanation |
| --- | --- |
| b1_takeoff = 80.4<br>b0_takeoff = 13.8 | Takeoff model constants (see Section 2). Units such that b1 * m^1.5/sqrt(p) is power in Watts. |
| if mode == "takeoff": ... elif mode == "landing": ... | Chooses which constants to use based on flight phase. |
| (m**1.5 / math.sqrt(p)) | Computes m1.5/ρ. Reflects induced-power scaling[1]. |
| * (h / V) | Multiplies power by time = distance/speed, yielding energy (J). |
| Emax - (energy_takeoff + energy_landing) | Remaining energy after vertical phases. |
| * Vcr / (... + b0_cruise) | Converts remaining energy to cruise distance: distance = speed × (energy / power). |

Figure 1 (conceptual): A block diagram of the flight phases and energy budget. (This figure would show three blocks for Takeoff, Cruise, Landing with arrows for energy flow. It would label vertical energy Eto and Eld from Task 1 feeding into the cruise distance calculation.)

+----------+      +-----------+      +----------+
      | Takeoff  |      |  Cruise   |      | Landing  |
      | Flight   |----->|  Flight   |<-----| Flight   |
      +----------+      +-----------+      +----------+
        ^     ^            ^    ^          ^      ^
        |     |            |    |          |      |
        |     +-- E_to     |    +-- E_cruise   E_ld -+
        |                                     |
      [b1_to, b0_to]* (h/V)           [b1_ld, b0_ld]* (h/V)

Table 2: Units for Inputs/Outputs

| Parameter | Symbol | Units | Description |
| --- | --- | --- | --- |
| Mass | m | kg | Drone mass |
| Density | p | kg/m³ | Air density (e.g. 1.225 at SL) |
| Height | h | m | Vertical climb or descent |
| Speed | V | m/s | Vertical (for climb/descent) or cruise speed |
| Energy | E | Joules (J) | Battery energy capacity |
| Distance | d | meters (m) | Horizontal range (output) |
| Constants | b1 | W·√ρ/kg^1.5 | Empirical induced-power factor |
| Constants | b0 | Watts (W) | Empirical offset power |

Figure 2 (suggested): A sensitivity plot of climb energy vs. mass. For example, with p=1.225, h=10 m, V=5 m/s, plot Eclimbm using the takeoff formula. This would illustrate the superlinear growth with mass (e.g. 2× mass increases energy by ~2.8×).[2][1]

(In an actual blog we would embed these figures. Here we describe them conceptually.)

## Frequently Asked Questions (FAQs)

- Why does the formula use m1.5/p?
This comes from rotorcraft momentum theory. The induced power needed to generate lift (thrust = mg) behaves like P∝TT/ρ. For constant rotor area, T=mg leads to P∝m3/2/ρ. Thus heavier drones or thinner air need much more power[1][2].

- What do the constants b1 and b0 represent?
They are empirical fit parameters. b1 scales the induced-power term and includes constants like gravity and rotor area. b0 is a base power (in Watts) accounting for profile drag and other losses. In takeoff/landing it’s fitted to data; in landing it’s even negative, indicating net power reduction during descent (regen) as fit by experiments[2].

- How are units consistent?
In total_energy, inside parentheses we compute a power (W). Then we multiply by time (h/V in seconds) to get energy (J). In calculate_distance, we divide energy by power (in the cruise phase) and multiply by speed to get distance. You can verify dimensionally: (J / W)·m/s = m.

- Why ignore cruise energy in total_energy?
total_energy is explicitly for vertical phases. The cruise energy is handled separately by calculate_distance. This keeps takeoff/landing logic simple. We assume cruise power is constant, so we don’t integrate it over distance – we just use available energy and cruise speed to get range.

- What if the model gives negative or zero range?
If Emax is too small to cover the vertical energy, the result can be zero or negative. The code doesn’t explicitly check this, so it may output a negative distance. In practice, one should check if Emax < Evert and handle as “not possible”.

- How do the tasks connect?
Task 1 supplies the energy/range model. Task 2 builds a dictionary of waypoints. Task 3 sorts waypoints by distance. Task 4 creates a Drone object that loads destinations (Task 2), updates a flight path (similar to Task 3 but using nearest-neighbor logic), and then uses Task 1’s logic (as calculate_energy and calculate_range methods) to check if legs are feasible. Essentially, Task 1 is the “engine” that tells Tasks 3–4 how far the drone can go.

## References

- Rotorcraft Induced Power: NDARC (NASA/Army Rotorcraft Handbook) describes the induced power formula: $P_{\rm ideal}=T\,v_i$ with vi=T/2ρA[1]. Substituting T=mg yields P∝m1.5/ρ.

- UAV Power Modeling: Dai et al. (2022) give a single-rotor UAV model: P=P0+1+k W3/2/2ρA[2], showing the W1.5/ρ term plus a constant profile power.

- Classic texts: (For further reading, see Johnson Helicopter Theory or Leishman Principles of Helicopter Aerodynamics for induced power theory.)

[1] NDARCTheory.tex

https://ntrs.nasa.gov/api/citations/20250010468/downloads/Johnson%20TP-202510468.pdf

[2] arxiv.org

https://arxiv.org/pdf/2209.04128